| Project Details (Capacity, Developer, State, Basin, Status) | Govt EIA / CIA GLOF Assessment | Academic / Independent GLOF Assessment | Flood Composition Modeled (Water vs. Sediment/Debris) | GLOF Risk (1-5) |
| --- | --- | --- | --- | --- |
| 1000 MW Pakal Dul (CVPPPL) J&K / Chenab / Under Construction | Assessed independently without a cumulative basin-wide GLOF analysis. Relies on standard dam-break routing [5]. | Identifies 197 glacial lakes (including Mundiksar) in the upper Chenab. Warns of extreme vulnerability due to fragile geology and seismicity [6]. | Govt/EIA: Water Only (Hydrodynamic discharge measured in cumecs). Academic: Highlights heavy sediment bulking and structural damage risks [5][6]. | 4 |
| 624 MW Kiru (CVPPPL) J&K / Chenab / Under Construction | Utilized standard dam-break routing. The EIA was criticized for poorly conducted, replicated impact predictions [5]. | Identifies exposure to GLOF-driven surges and cascading sediment loads from upstream glacial lakes [5][6]. | Govt/EIA: Water Only. Academic: Emphasizes extreme sediment loads and debris [6]. | 4 |
| 540 MW Kwar (CVPPPL) J&K / Chenab / Under Construction | Found to be a replica of the Kiru EIA, utilizing generic dam-break routing without site-specific risk modeling [5]. | Highlights risk from high-altitude lakes destabilizing and funneling down the narrow gorge [5]. | Govt/EIA: Water Only. Academic: Highlights mass-wasting and hyper-concentrated sediment loads [5]. | 4 |
| 210 MW Purthi (SJVN) HP / Chenab / Under Construction | Relies on individual clearances without a cumulative basin-level sediment assessment [5]. | Warns of cascading mass-wasting vulnerabilities in the geologically fragile Chenab basin headwaters [5]. | Govt/EIA: Water Only. Academic: Highlights severe risk of hyper-concentrated flows [5]. | 4 |
| 260 MW Sach Khas (SJVN) HP / Chenab / Cleared | Clearances ignore cumulative debris impacts across the Chenab cascade, utilizing standard FRL calculations [5]. | Warns that extreme weather and expanding lakes (e.g., Ghepan Gath) increase the cascade's vulnerability [5]. | Govt/EIA: Water Only. Academic: Highlights hyper-concentrated sediment risks [5]. | 4 |
| 500 MW Dugar (NHPC) HP / Chenab / Under Construction | Clearances bypass cumulative basin-level sediment modeling [5]. | Emphasizes the lack of cumulative impact assessments for the entire Chenab cascade facing direct GLOF tracks [5]. | Govt/EIA: Water Only. Academic: Highlights hyper-concentrated sediment risks [5]. | 4 |
| 120 MW Miyar (Hindustan Power/SJVN) HP / Chenab / Cleared | EIA ignores downstream cascade vulnerabilities [5]. | Identifies severe vulnerability of cascade projects situated directly on historical GLOF tracks [5]. | Govt/EIA: Water Only. Academic: Highlights hyper-concentrated sediment risks [5]. | 4 |
| 520 MW Tapovan Vishnugad (NTPC) Uttarakhand / Dhauliganga / Under Construction | Initial clearances relied on Probable Maximum Flood data; failed to account for hanging glacier collapses [1]. | Mapped the exact trajectory of roughly 27 million cubic meters of rock and ice that destroyed the project in 2021 [1]. | Govt/EIA: Water Only (Pre-disaster). Academic: Modeled the exact density of sediment, rock, and ice (hyper-concentrated flow) [1]. | 5 |
| 180 MW Bajoli Holi (GMR Energy) HP / Ravi / Commissioned | Spillway capacity approved based on standard Inflow Design Flood calculations [4]. | MIKE 11 hydrodynamic modeling of upstream lake GL-52 emphasizes the need to account for combined GLOF risks [4]. | Govt/EIA: Water Only. Academic: Calculates transport of considerable sediment amounts altering floodplains [4]. | 3 |
| 1200 MW Teesta III (Sikkim Urja) Sikkim / Teesta / Destroyed | Historically evaluated as a hydraulic wave. Post-disaster rebuild mandated a 19,946 cumecs spillway [3]. | Assessed the transition of a Himalayan GLOF into a severe transborder debris flow, predicting lake expansion [3]. | Govt/EIA: Water Only. Academic: Modeled pre-existing sediment ingestion amplifying the flood by three orders of magnitude [3]. | 5 |
| 510 MW Teesta V (NHPC) Sikkim / Teesta / Damaged | Historically evaluated as a hydraulic wave [3]. | Assessed GLOF transition into a severe transborder debris flow and moraine breach [3]. | Govt/EIA: Water Only. Academic: Modeled pre-existing sediment ingestion [3]. | 5 |
| 2000 MW Subansiri Lower (NHPC) Arunachal / Brahmaputra / Under Construction | Modeled dam-break of upstream lakes by routing clear-water discharge [2]. | Focuses on estimating regional exposure to GLOFs and uncertainty in the transboundary energy-water nexus [2]. | Govt/EIA: Water Only. Academic: Highlights massive upstream landslide dams introducing geological material [2]. | 4 |
| 2880 MW Dibang Multipurpose (NHPC) Arunachal / Brahmaputra / Cleared | Modeled dam-break routing clear-water discharge [2]. | Focuses on estimating regional exposure to extreme transboundary geomorphic risks [2]. | Govt/EIA: Water Only. Academic: Highlights massive upstream landslide dams introducing geological material [2]. | 4 |

References

[1] Shugar, D. H., et al. (2021). A massive rock and ice avalanche caused the 2021 disaster at Chamoli, Indian Himalaya. Science, 373(6552), 300-306. DOI: 10.1126/science.abh4455.

[2] Schwanghart, W., et al. (2016). Uncertainty in the Himalayan energy–water nexus: estimating regional exposure to glacial lake outburst floods. Environmental Research Letters, 11(7), 074005. DOI: 10.1088/1748-9326/11/7/074005.

[3] Sattar, A., et al. (2022). Transition of a small Himalayan glacier lake outburst flood to a giant transborder flood and debris flow. Scientific Reports, 12, 12450. DOI: 10.1038/s41598-022-15549-y.

[4] Bhatt, A., et al. (2024). Comprehensive analysis of hydrological and glacial dynamics in the Himalayas: addressing glacial lake outburst flood risks in hydropower development. (Research modeling for Bajoli-Holi and GL-52).

[5] South Asia Network on Dams, Rivers and People (SANDRP). (2014–2025). Independent assessments, EIA critiques, and cascade impact analyses of Chenab and Ravi Basin hydropower projects.

[6] Jammu & Kashmir District Disaster Management Authority / Greater Kashmir Analysis (2026). GLOF Risk Management Plan, 2024